Design Rules

For MFG boards, maybe Sparkfun.dru

DIY boards

Should be able to use MTS.dru

Clearance

Wire to Wire 24mil

Wire to Pad 16mil

Wire to via 24mil

Size

Min Width 16mil

Mid Drill 12mil?

Routing wires Width 0.016	Drill: 0.03149

Sometimes need to decrease Clearance

Wire to Pad 12mil

To run traces between header pins

Then I set it back to 16mil

To run polygon GND

This will give clearance error but should allow more clearance for ground

plane

Toner Transfer:

NOTE: If you want traces on top, be sure to change autoroute and select 1-TOP below instead of 16-Bottom

TRACES on Bottom

Eagle: Under  options - user interface - Background White

Eagle: In the board editor, there is a little icon called Layer Settings (Show/Hide Layers). Click on it, <Hide Layers>, then click on the Nr

16-Bottom

17-Pads

18-Vias

20-Dimensions

45-Holes

Now, just the bottom traces should be displayed.

TRACES on Top

Eagle: Under  options - user interface - Background White

Eagle: In the board editor, there is a little icon called Layer Settings (Show/Hide Layers). Click on it, <Hide Layers>, then click on the Nr

1-Top

17-Pads

18-Vias

20-Dimensions

45-Holes

Now, just the top traces should be displayed. (remember to invert)

Select <File><Export><Image>

<Monochrome>

2400DPI.

<Browse> Select the directory and file name (e.g., W4Traces) that you want.

Click <OK> to export as image.

Eagle exports as a PNG file.

SILK

For Silk, select layers:

17-Pads

18-Via

20-Dimension

21-tPlace

25-tNames

27-tValues

45-Holes

Select 2400 DPI, Monochrome

Eagle exports as a PNG file.

Jumper Wires

For Jumper wires, select layers:

16-Bottom

17-Pads

18-Via

21-tPlace

25-tNames

Select 2400 DPI, Monochrome

Eagle exports as a PNG file.

TRACES

Make Negative: Should already be negative

******************************************************************************************

NOTE: If you want traces on top, be sure to change autoroute and select 1-TOP below instead of 16-Bottom, do this also

<Image><Rotate><Flip Horizontal>

SILK

Invert:

Open Image with PhotoShopElements

<Select> All

<Image><Rotate><Flip Horizontal {Lettering is mirrored}

<File><SaveAs>.... INV.png

SAMSUNG TONER

Toner Transfer Vinyl Sticker

The reason, I make multiple copies is that my laser printer will only print full 8.5 x 11 sheets. So printing a single copy of a little PCB would waste a lot of paper (and toner?). So I decided to make the traces and the silk at the same time. This also leaves the option of making several PCBs at once.

Make multiple copies:

Open the inverted images with Photoshop Elements

Click on Photoshop Elements, select Photo Editor

<File><New> <Blank File>

Create new base file, <File><New>  7.9x10

Resolution 2400

Grayscale

<File><Open>   Find Silk

Invert:

<Select> All

<Image><Rotate><Flip Horizontal {Lettering is mirrored}

<File><SaveAs>.... INV.png

<File><Open>   Find Traces

Top Menu, select Expert

Bottom menu Layout

<Arrange> Select the grid so that the display will show all files

Select Inverted traces file, <Select><All><Edit><Copy>

Select Base file, <Edit><Paste>

Use Mouse to select Move tool, upper left corner

Move the copied image with mouse

<Ctrl-V> to repeat pastes

Repeat the process with the Silk image

For the size of PCBs I've been making, I usually make 3 copies of each

Keep images separated so it's easier to cut them and leave a border

<File><Save As> <Format><JPEG>  I've been naming it MultiTrace

Print using Photoshop Elements

REMEMBER: CHANGE TO SAMSUNG TONER CARTRIDGE

\\SECONDARY-PC\Samsung ML-1865W Setup

Basic: Quality: Best

Paper: Scaling Options: 100%

Graphics: Darken Text, All Text Black

Fine Edge

Toner Saver: Off

Darkness: Dark

Select Print size 8” x 10”

On the paper in the sheet feeder, mark a corner of the top piece of paper. I mark an 'LR' for lower right corner. {This is so the same piece of paper can be fed back into the printer with the vinyl attached.

For some reason in Photoshop Elements, I have to close the two inverted files in order to print the multi file

<File><Print>

Make sure the correct printer is selected

Make sure it's set for actual size

PRINT

Check the printout for any errors. Make sure the dimensions are correct.

Cut pieces of vinyl for the traces and the silk with a little margin for error.

Remove the paper backing and carefully place over the traces and images on the printed paper. You should have a little overlap.

Stick the printed paper (with vinyl) back into printer paper feed in same orientation using your mark

PRINT again.

Touchup: Sharpie

Cutout the traces printed onto the vinyl with a generous border. I like to include the dimensions as it gives you some reference lines.

Cut a piece of copperclad(PCB) to size. I use .031" copper clad.

Chamfer the edges of copperclad.

Clean the copperclad

I use Scotchbrite and acetone.

Tape the traces to the copper side so they are aligned. I use scotch tape.

Trim the trace paper, so there's just a little for the tape to hold.

TonerTransfer

I use a modified laminator.

Turn it on and heat it up.

Run the PCB through several times. I've been doing about 10 passes.

Put PCB in cold water.

Peel off paper and tape from PCB.

My scotch tape, leaves a sticky residue, but I ignore it until after etching.

Check PCB for problems.

If major, clean off traces with acetone and reprint.

If minor, touchup with red Sharpie. My friend OffTheRails2010 suggested red.

Red works better than black.

Etch the PCB

Mix up your favorite etchant in a well ventilated area. I use:

Etchant:

Two parts hydrogen peroxide (3%)

One part Muriatic acid (31%)

Place PCB in etchant and wait for etching

When done, rinse in cold water.

Clean off traces and sticky tape residue with acetone

Silk

Cut printed silk vinyl larger than PCB.

I like to drill two of the holes near opposite corners in PCB and in silk vinyl.

Match up the silk on the blank side of the PCB.

Stick some resistor leads through the silk and the PCB to align the two.

Scotch tape the two together then remove the resistor leads.

Trim the silk, leaving just a little for the tape to hold.

Heat up laminator

Run through several times.

Cool it off in water.