; STROBOSCOPE 2011 07 04

; To compile cvasm16.exe strobosc.src

include '12f675.inc'

_BODEN_OFF           EQU        3FBFh

_MCLRE_OFF           EQU        3FDFh

_INTRC_OSC_NOCLKOUT  EQU        3FFCh

_WDT_ON              EQU        3FFFh

FUSES _BODEN_OFF

FUSES _MCLRE_OFF

FUSES _INTRC_OSC_NOCLKOUT

FUSES _WDT_ON

; Define some variables in the general purpose FILE

; register area.

STATUS       EQU        0003H

GPIO         EQU        0005H

ADCON0       EQU        001FH

CMCON        EQU        0019H

OPTION_REG   EQU        0081H

TRISIO       EQU        0085H

WPU          EQU        0095H

ANSEL        EQU        009FH

CLOCK1       EQU        0021H; -- CLOCK   ----

POTMSB       EQU        0022H; AD CH 3

; ------ MAIN PROGRAM -------------

CALL INIT         ; INITIALIZATION

STROBE

BSF GPIO,0             ; TURN ON LED

CALL DELAY100US        ;

BCF GPIO,0             ; TURN OFF LED

CALL DELAY100US;

CALL DELAY100US;

CALL DELAY100US;

CALL GETPOTAD2

OFFLOOP

CALL DELAY100US;

CLRWDT        ; REFRESH WATCHDOG TIMER

MOVF POTMSB,1 ; TEST IF POTMSB IS ZERO

BTFSC STATUS,2; TEST STATUS FOR ZERO

GOTO STROBE   ; =ZERO, NOT TO DECREMENT

DECF POTMSB,1;

GOTO OFFLOOP     ; 3 CLOCK CYCLES

GOTO STROBE

;------------------------------------------------------------------------

GETPOTAD2

MOV ADCON0,#00001001B  ; LEFT JUSTIFIED, CH 2

BSF ADCON0,1           ; START AD CONVERSION

WAITPOTAD2

BTFSC ADCON0,1     ; DATA READY?

GOTO WAITPOTAD2    ;

MOV POTMSB,ADRESH  ;

RET                ;

;------------------------------------------------------------------------

;-------------------- INIT --------------------------

INIT

BCF   STATUS,5

CLRF  GPIO

MOVLW 00000111B ;

MOVWF  CMCON          ; DISABLED COMPARATOR

BSF   STATUS,5        ; PAGE 2

CLRF ANSEL            ; DIGITAL IO

MOVLW 00100010B     ; weak pull up gp1, GP5

MOVWF WPU             ;

MOVLW 00111100B     ;

MOVWF  ANSEL          ; SET AN3, AN2 AS A/D + INTERNAL RC

MOVLW 00001000B     ;

MOVWF OPTION_REG          ;

MOV ADCON0,#00000000B ;LEFT JUSTIFIED, AD REF=VDD, START AD3 FIRST

MOVLW 00111110B    ; GP0 AS OUTPUT, GP1,2,3,4,5 as input

MOVWF TRISIO          ;

MOVLW 10000000B     ; CENTER FREQUENCY

MOVWF OSCCAL          ;

BCF   STATUS,5

CLRF   INTCON          ;

CLRWDT                ;

RET

;------------------- END INIT --------------------------

;-------------------DELAY100US-------------------------------------

DELAY100US

MOV CLOCK1, #36   ;

D100LOOP

DECFSZ CLOCK1,1   ;

GOTO D100LOOP     ;

RET

;-------------- END DELAY100US ---------------------