'

'		Tic Tac Tunes -  (c) Andy Stansfield 15/7/2009 (Andy Gadget)

'		Released under Attribution - Non commercial - No derivs license

'

'

'		This is the first released version, but is undergoing a major re-write

'		at the moment. The PicAxe 08m is a brilliant little chip, but a program

' 		like this needs every byte of its 256 byte program/data memory.

'		Every byte is precious. The code itself is a structural abomination (and

'		going to get worse), but writing 'pretty' code takes too much space.

'		The 08m has the option of running at 8MHz, but this messes up the octaves.

'

'

'		Note : If you program this, it powers up in 'sleep' mode and wakes every

'		4 seconds to see if anything is happening.  It may not be awake when the

'		PC tries to communicate.  Simplest way around is to turn it on first -

'		Probe onto the control blob until it goes 'beep'!

'		It also powers up after programming in sleep mode and reset, but will retain

'		its settings (scale, tempo) after inactivity time-out and re-awaken.

'		(I will comment the code after the re-write and when I get time #;¬)

#picaxe 08m

disablebod

data (89,89,89,87,84,82,80,73,71,68,66,64,105,103,100,98,96,96) 'Pentatonic 1/8 notes

data (90,88,85,84,83,81,74,72,69,68,67,65,106,104,101,100,99,97) 'Blues scale 1/8

'data (69,68,67,66,65,64,107,106,105,104,103,102,101,100,99,98,97,96) 'Chromatic 1/8

symbol LED		=	1

symbol Perc		=	b7

symbol Tmp1		=	b6

symbol Num		=	b5

symbol Note		=	b4

symbol Tempo	=	b3

symbol Scale	=	b2

symbol Key		=	b1

symbol ADC 		= 	b0

tempo	=	4

scale = 0

pwrsave:

input 4

do

sleep 4

loop until pin4 = 1

tune 0,4,(84,89)

main:

do

readadc 4, adc

inc w3

if w3 = 20000 then

tune 0,4,(89,84)

goto pwrsave

end if

loop until adc > 5

if adc > 126 then

if adc >253 then goto settempo

perc = adc - 128 / 13

on perc goto null,hibel,midbel,lobel,scrape,shake,trill,cowbell,snare

else

tmp1 = adc * 155 / 1000 min 0 max 17

key = scale * 18 + tmp1

read key,note

tune 0,tempo,(note)

end if

goto main

null:

goto main

snare:

sound 2,(254,3,240,3,230,8)

goto keyup

cowbell:

sound 2,(120,1,110,2)

goto keyup

trill:

sound 2,(122,1,10,2,122,1,10,2)

goto keyup

shake:

sound 2,(200,1,255,1)

goto keyup

scrape:

sound 2,(250,3,251,3,252,3,254,5)

goto keyup

lobel:

sound 2,(120,4)

goto keyup

midbel:

sound 2,(122,4)

goto keyup

hibel:

sound 2,(124,4)

goto keyup

keyup:

do

readadc 4	, adc

loop until adc<5

goto main

settempo:

tempo = tempo //8+1

high led

pause 5

low led

pause 400

if tempo=1 then

scale = scale //1+1

high led

pause 500

low led

end if

goto main